# 1.0 Purpose

This procedure describes tablet compression and release testing.

It applies to all commercial batches manufactured at Site B.

# 2.0 Scope

Covers in-process control, finished product testing, and release.

Excludes stability testing, which is covered by SOP-QA-118.

# 3.0 Equipment

Compression is performed on the rotary press listed below.

| Equipment | Model | Capacity |
| --- | --- | --- |
| Rotary Tablet Press | Fette 3090 | 100,000 tablets/hour |

# 4.0 Sampling

Samples are drawn at the start, middle, and end of each run.

A retained sample of 30 tablets is held for each batch.

Sampling tools are cleaned between batches.

| Sampling Point | Quantity | Frequency |
| --- | --- | --- |
| Start of run | 10 tablets | Once per batch |
| Middle of run | 10 tablets | Once per batch |
| End of run | 10 tablets | Once per batch |

# 5.0 Procedure

Compression force shall be maintained at 18 kN plus or minus 2 kN.

CAUTION: verify press calibration before each batch.

Record the force reading every 30 minutes during the run.

# 6.0 Acceptance Criteria

Assay shall be 95.0 percent to 105.0 percent of label claim.

| Parameter | Limit | Method |
| --- | --- | --- |
| Assay | 95.0% to 105.0% of label claim | HPLC |
| Dissolution | Not less than 80% in 30 minutes | USP <711> |
| Content Uniformity | AV less than or equal to 15 | USP <905> |

# 7.0 Deviations

Deviations are recorded in the quality management system.

| Deviation Type | Severity |
| --- | --- |
| Equipment malfunction | Major |
| Documentation error | Minor |

| Shipping Lane | Carrier | Transit Time |
| --- | --- | --- |
| Site B to Warehouse 4 | Cold chain courier | 48 hours |
| Warehouse 4 to Depot | Ambient freight | 72 hours |

# 8.0 Training

Operators are trained before independent operation.

Training records are held by the QA department.

| Course | Frequency | Duration |
| --- | --- | --- |
| GMP Basics | Annual | 2 hours |
| Equipment Operation | Biennial | 4 hours |

# 9.0 Records

Batch records are retained for seven years.

| Record Type | Retention Period | Storage Location |
| --- | --- | --- |
| Batch Record | 7 years | Archive Room A |
| Equipment Log | 5 years | Archive Room B |

# 10.0 Definitions

In-process control means testing performed during manufacture.

Retained sample means material held for future examination.

Release means the formal decision to make a batch available.

| Term | Definition | Reference |
| --- | --- | --- |
| In-process control | Testing performed during manufacture | QA-101 |
| Retained sample | Material held for future examination | QA-102 |

# 11.0 Approval

This procedure is approved by the QC Manager.

| Approved By | Reviewed By | Date |
| --- | --- | --- |
| QC Manager | Production Manager | 2024-01-15 |

# 12.0 References

Refer to the equipment manual for calibration intervals.

| Reference | Number | Revision |
| --- | --- | --- |
| Equipment Manual | EQ-MAN-01 | Rev 3, dated 10 Jan 2022 |

Refer to Equipment Manual EM-004, Rev 3, for calibration procedure.